# SPIM Instruction Set

# Instructions and PseudoInstructions

This list of MIPS instructions and pseudoinstructions is not complete. Notably missing are some of the Floating Point and other coprocessor instructions.

• - Indicates an actual MIPS instruction. Others are SPIM PseudoInstructions.

Instruction	Function

•	add	Rd, Rs, Rt	Rd = Rs + Rt (signed)

•	addu	Rd, Rs, Rt	Rd = Rs + Rt (unsigned)

•	addi	Rd, Rs, Imm	Rd = Rs + Imm (signed)

•	sub	Rd, Rs, Rt	Rd = Rs - Rt (signed)

•	subu	Rd, Rs, Rt	Rd = Rs - Rt (unsigned)

•	div	Rs, Rt	lo = Rs/Rt, hi = Rs mod Rt (integer division, signed)

•	divu	Rs, Rt	lo = Rs/Rt, hi = Rs mod Rt (integer division, unsigned)

div	Rd, Rs, Rt	Rd = Rs/Rt (integer division, signed)

divu	Rd, Rs, Rt	Rd = Rs/Rt (integer division, unsigned)

rem	Rd, Rs, Rt	Rd = Rs mod Rt (signed)

remu	Rd, Rs, Rt	Rd = Rs mod Rt (unsigned)

mul	Rd, Rs, Rt	Rd = Rs * Rt (signed)

•	mult	Rs, Rt	hi, lo = Rs * Rt (signed, hi = high 32 bits, lo = low 32 bits)

•	multu	Rd, Rs	hi, lo = Rs * Rt (unsigned, hi = high 32 bits, lo = low 32 bits)

•	and	Rd, Rs, Rt	Rd = Rs • Rt

•	andi	Rd, Rs, Imm	Rd = Rs • Imm

neg	Rd, Rs	Rd = -(Rs)

•	nor	Rd, Rs, Rt	Rd = (Rs + Rt)’

not	Rd, Rs	Rd = (Rs)’

•	or	Rd, Rs, Rt	Rd = Rs + Rt

•	ori	Rd, Rs, Imm	Rd = Rs + Imm

•	xor	Rd, Rs, Rt	Rd = Rs Rt

•	xori	Rd, Rs, Imm	Rd = Rs Imm

•	sll	Rd, Rt, Sa	Rd = Rt left shifted by Sa bits

•	sllv	Rd, Rs, Rt	Rd = Rt left shifted by Rs bits

•	srl	Rd, Rs, Sa	Rd = Rt right shifted by Sa bits

•	srlv	Rd, Rs, Rt	Rd = Rt right shifted by Rs bits

move	Rd, Rs	Rd = Rs

•	mfhi	Rd	Rd = hi

•	mflo	Rd	Rd = lo

li	Rd, Imm	Rd = Imm

•	lui	Rt, Imm	Rt[31:16] = Imm, Rt[15:0] = 0

•	lb	Rt, Address(Rs)	Rt = byte at M[Address + Rs] (sign extended)

•	sb	Rt, Address(Rs)	Byte at M[Address + Rs] = Rt (sign extended)

•	lw	Rt, Address(Rs)	Rt = word at M[Address + Rs]

•	sw	Rt, Address(Rs)	Word at M[Address + Rs] = Rt

•	slt	Rd, Rs, Rt	Rd = 1 if Rs < Rt, Rd = 0 if Rs Rt (signed)

•	slti	Rd, Rs, Imm	Rd = 1 if Rs < Imm, Rd = 0 if Rs Imm (signed)

•	sltu	Rd, Rs, Rt	Rd = 1 if Rs < Rt, Rd = 0 if Rs Rt (unsigned)

•	beq	Rs, Rt, Label	Branch to Label if Rs = Rt

beqz	Rs, Label	Branch to Label if Rs = 0

bge	Rs, Rt, Label	Branch to Label if Rs Rt (signed)

•	bgez	Rs, Label	Branch to Label if Rs 0 (signed)

•	bgezal	Rs, Label	Branch to Label and Link if Rs Rt (signed)

bgt	Rs, Rt, Label	Branch to Label if Rs > Rt (signed)

bgtu	Rs, Rt, Label	Branch to Label if Rs > Rt (unsigned)

•	bgtz	Rs, Label	Branch to Label if Rs > 0 (signed)

ble	Rs, Rt, Label	Branch to Label if Rs Rt (signed)

bleu	Rs, Rt, Label	Branch to Label if Rs Rt (unsigned)

•	blez	Rs, Label	Branch to Label if Rs 0 (signed)

•	bgezal	Rs, Label	Branch to Label and Link if Rs 0 (signed)

•	bltzal	Rs, Label	Branch to Label and Link if Rs < 0 (signed)

blt	Rs, Rt, Label	Branch to Label if Rs < Rt (signed)

bltu	Rs, Rt, Label	Branch to Label if Rs < Rt (unsigned)

•	bltz	Rs, Label	Branch to Label if Rs < 0 (signed)

•	bne	Rs, Rt, Label	Branch to Label if Rs Rt

bnez	Rs, Label	Branch to Label if Rs 0

•	j	Label	Jump to Label unconditionally

•	jal	Label	Jump to Label and link unconditionally

•	jr	Rs	Jump to location in Rs unconditionally

•	jalr	Label 	Jump to location in Rs and link unconditionally

# System I/O Services:   syscall

| Service | Code in $v0 | Argument(s) | Result(s) |
| --- | --- | --- | --- |
| Print Integer | 1 | $a0 = number to be printed |  |
| Print Float | 2 | $f12 = number to be printed |  |
| Print Double | 3 | $f12 = number to be printed |  |
| Print String | 4 | $a0 = address of string in memory |  |
| Read Integer | 5 |  | number returned in $v0 |
| Read Float | 6 |  | number returned in $f0 |
| Read Double | 7 |  | number returned in $f0 |
| Read String | 8 | $a0 = address of input buffer in memory |  |
|  |  | $a1 = length of buffer (n) |  |
| Sbrk | 9 | $a0 = amount | address in $v0 |
| Exit | 10 |  |  |

# Registers

By convention, many MIPS registers have special purpose uses. To help clarify this, SPIM defines aliases for each register that represent its purpose. The following table lists these aliases and the commonly accepted uses for the registers.

| Register | Number | Usage |
| --- | --- | --- |
| zero | 0 | Constant 0 |
| at | 1 | Reserved for the assembler |
| v0 | 2 | Used for return values from function calls. |
| v1 | 3 |  |
| a0 | 4 | Used to pass arguments to procedures and functions. |
| a1 | 5 |  |
| a2 | 6 |  |
| a3 | 7 |  |
| t0 | 8 | Temporary (Caller-saved, need not be saved by called procedure) |
| t1 | 9 |  |
| t2 | 10 |  |
| t3 | 11 |  |
| t4 | 12 |  |
| t5 | 13 |  |
| t6 | 14 |  |
| t7 | 15 |  |
| s0 | 16 | Saved temporary (Callee-saved, called procedure must save and restore) |
| s1 | 17 |  |
| s2 | 18 |  |
| s3 | 19 |  |
| s4 | 20 |  |
| s5 | 21 |  |
| s6 | 22 |  |
| s7 | 23 |  |
| t8 | 24 | Temporary (Caller-saved, need not be saved by called procedure) |
| t9 | 25 |  |
| k0 | 26 | Reserved for OS kernel |
| k1 | 27 |  |
| gp | 28 | Pointer to global area |
| sp | 29 | Stack pointer |
| fp | 30 | Frame pointer |
| ra | 31 | Return address for function calls. |

| Decimal | Hex | Octal | Binary | Decimal | Hex | Octal | Binary |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0000 | 8 | 8 | 10 | 1000 |
| 1 | 1 | 1 | 0001 | 9 | 9 | 11 | 1001 |
| 2 | 2 | 2 | 0010 | 10 | A | 12 | 1010 |
| 3 | 3 | 3 | 0011 | 11 | B | 13 | 1011 |
| 4 | 4 | 4 | 0100 | 12 | C | 14 | 1100 |
| 5 | 5 | 5 | 0101 | 13 | D | 15 | 1101 |
| 6 | 6 | 6 | 0110 | 14 | E | 16 | 1110 |
| 7 | 7 | 7 | 0111 | 15 | F | 17 | 1111 |

# Floating Point Instructions and PseudoInstructions

The MIPS floating point coprocessor is coprocessor number 1. It operates on single precision (32-bit) and double precision (64-bit) floating point numbers. This coprocessor has its own registers, numbered $f0-$f31. Even numbered registers are used for double precision operations.

• - Indicates an actual MIPS instruction. Others are SPIM pseudo instructions.

Instruction	Function

•	abs.d	FRdest, FRsrc	FRdest = | FRsrc |

•	abs.s	FRdest, FRsrc	FRdest = | FRsrc |

•	add.d	FRdest, FRsrc1, FRsrc2	FRdest = FRsrc1 + FRsrc2

•	add.s	FRdest, FRsrc1, FRsrc2	FRdest = FRsrc1 + FRsrc2

•	c.eq.d	FRsrc1, FRsrc2	Set flag on FRsrc1 = FRsrc2

•	c.eq.s	FRsrc1, FRsrc2	Set flag on FRsrc1 = FRsrc2

•	c.le.d	FRsrc1, FRsrc2	Set flag on FRsrc1  FRsrc2

•	c.le.s	FRsrc1, FRsrc2	Set flag on FRsrc1  FRsrc2

•	c.lt.d	FRsrc1, FRsrc2	Set flag on FRsrc1 < FRsrc2

•	c.lt.s	FRsrc1, FRsrc2	Set flag on FRsrc1 < FRsrc2

•	cvt.d.s	FRdest, FRsrc	Double ← Single

•	cvt.d.w	FRdest, FRsrc	Double ← Integer

•	cvt.s.d	FRdest, FRsrc	Single ← Double

•	cvt.s.w	FRdest, FRsrc	Single ← Integer

•	cvt.w.d	FRdest, FRsrc	Integer ← Double

•	cvt.w.s	FRdest, FRsrc	Integer ← Single

•	div.d	FRdest, FRsrc1, FRsrc2	FRdest = FRsrc1 / FRsrc2

•	div.s	FRdest, FRsrc1, FRsrc2	FRdest = FRsrc1 / FRsrc2

l.d	FRdest, address	FRdest = address

l.s	FRdest, address	FRdest = address

•	mov.d	FRdest, FRsrc	FRdest = FRsrc

•	mov.s	FRdest, FRsrc	FRdest = FRsrc

•	mul.d	FRdest, FRsrc1, FRsrc2	FRdest = FRsrc1 * FRsrc2

•	mul.s	FRdest, FRsrc1, FRsrc2	FRdest = FRsrc1 * FRsrc2

•	neg.d	FRdest, FRsrc	FRdest =  - FRsrc

•	neg.s	FRdest, FRsrc	FRdest =  - FRsrc

s.d	FRdest, address	address ← FRdest

s.s	FRdest, address	address ← FRdest

•	sub.d	FRdest, FRsrc1, FRsrc2	FRdest = FRsrc1 – FRsrc2

•	sub.s	FRdest, FRsrc1, FRsrc2	FRdest = FRsrc1 – FRsrc2

•	bc1t	Label	Branch conditional; if flag is True; to Label

•	bc1f	Label	Branch conditional; if flag is False; to Label

(1 denotes coprocessor 1, the floating point coprocessor)

•	mfc1	Rdest, FPsrc	Move contents of register FRsrc to CPU register Rdest

•	mtc1	Rsrc, FPdest	Move contents of CPU register Rsrc to register FPdest